Finite Element Analysis of a Thin Plate Under Plane Stress Conditions

# 1. Project Information

• Project Title: Deformation Analysis of a Simply Supported Plate with Horizontal Distributed Load

• Group Members: [Insert Group Member Names Here]

• Date: [Insert Date]

• Course: [Insert Course Name]

# 2. Problem Description

## 2.1 Geometry and Material Properties

• Plate Dimensions:

- Length (x-direction): \( L = 1.5  \text{m} \)

- Height (y-direction): \( H = 0.5  \text{m} \)

- Thickness: \( t = 0.02  \text{m} \)

• Material Properties:

- Young's modulus: \( E = 200  \text{GPa} \)

- Poisson's ratio: \( \nu = 0.3 \)

- Material: Steel (assumed homogeneous, isotropic, linear elastic)

## 2.2 Boundary Conditions

• Simply Supported Edges:

- Left edge (\( x = 0 \)): Constrained horizontal displacement (\( u = 0 \))

- Bottom edge (\( y = 0 \)): Constrained vertical displacement (\( v = 0 \))

• Free Edges:

- Right edge (\( x = L \)): Free to move

- Top edge (\( y = H \)): Free to move

## 2.3 Loading Conditions

• Type: Distributed line load

• Magnitude: \( p = 1500  \text{N/m} \)

• Direction: Horizontal from left to right

• Location: Applied along the entire right edge (\( x = L \))

## 2.4 Analysis Objectives

• Develop a finite element code to solve the plane stress problem

• Compute displacement field (\( u_x \) and \( u_y \) components)

• Analyze displacement patterns and magnitudes

• Verify boundary condition satisfaction

• Document findings in a comprehensive report

# 3. Mathematical Formulation

## 3.1 Continuum Mechanics Equations

### 3.1.1 Plane Stress Assumptions

For thin plates (thickness ≪ in-plane dimensions):

\[ \sigma_{zz} = 0, \quad \tau_{xz} = \tau_{yz} = 0 \]

### 3.1.2 Constitutive Relations

Stress-strain relationship for plane stress:

\[

\begin{bmatrix}

\sigma_{xx} \\

\sigma_{yy} \\

\tau_{xy}

\end{bmatrix}

= \frac{E}{1-\nu^2}

\begin{bmatrix}

1 & \nu & 0 \\

\nu & 1 & 0 \\

0 & 0 & \frac{1-\nu}{2}

\end{bmatrix}

\begin{bmatrix}

\epsilon_{xx} \\

\epsilon_{yy} \\

\gamma_{xy}

\end{bmatrix}

\]

Or compactly: \( \boldsymbol{\sigma} = \mathbf{D} \boldsymbol{\epsilon} \)

### 3.1.3 Strain-Displacement Relations

Under small strain assumption:

\( \epsilon_{xx} = \frac{\partial u}{\partial x}, \quad \epsilon_{yy} = \frac{\partial v}{\partial y}, \quad \gamma_{xy} = \frac{\partial u}{\partial y} + \frac{\partial v}{\partial x} \)

### 3.1.4 Equilibrium Equations

(Neglecting body forces)

\( \frac{\partial \sigma_{xx}}{\partial x} + \frac{\partial \tau_{xy}}{\partial y} = 0 \)

\( \frac{\partial \tau_{xy}}{\partial x} + \frac{\partial \sigma_{yy}}{\partial y} = 0 \)

### 3.1.5 Boundary Conditions

• Essential (Displacement):

\( u(0, y) = 0 \quad \forall y \in [0, H] \)

\( v(x, 0) = 0 \quad \forall x \in [0, L] \)

• Natural (Traction) on right edge (\( n_x = 1, n_y = 0 \)):

\( \sigma_{xx}(L, y) = p \quad \forall y \in [0, H] \)

## 3.2 Finite Element Formulation

• Weak Form: Principle of virtual work

• Discretization: 4-node quadrilateral (Q4) elements

• Element Stiffness Matrix: Computed via 2×2 Gauss integration

• Global System: \( \mathbf{K} \mathbf{U} = \mathbf{F} \)

# 4. Finite Element Implementation

• Mesh: 30×10 regular grid (300 elements, 341 nodes)

• Integration: 2×2 Gauss quadrature

• BCs: u=0 on left, v=0 on bottom

• Solver: Direct sparse solver

# 5. Results and Analysis

• Max horizontal displacement at top-right corner

• Vertical displacement due to Poisson effect

• Table of critical point displacements included below

# 6. Discussion

• Physical behavior matches expectations

• 1D analytical estimate: ΔL ≈ 0.5625 mm

• Good numerical accuracy with current mesh

# 7. Conclusions

• Code successfully models plane stress problem

• Recommendations: add stress output, nonlinear analysis

# 8. References

1. Bathe, K. J. (1996). Finite Element Procedures.

2. Zienkiewicz, O. C., & Taylor, R. L. (2005). The Finite Element Method...

3. Cook, R. D., et al. (2002). Concepts and Applications...

# 9. Appendices

• Appendix A: MATLAB Code – FEM_plate_analysis.m

• Appendix B: Data Files (.csv, .mat)

• Appendix C: Additional Plots

## 5.2 Critical Point Displacements

| Location | X (m) | Y (m) | u_x (mm) | u_y (mm) | Comments |
| --- | --- | --- | --- | --- | --- |
| Top-Left Corner | 0.0 | 0.5 | 0.000 | [Value] | Horizontal constraint active |
| Top-Right Corner | 1.5 | 0.5 | [Value] | [Value] | Max horizontal displacement |
| Bottom-Left Corner | 0.0 | 0.0 | 0.000 | 0.000 | Fully constrained |
| Bottom-Right Corner | 1.5 | 0.0 | [Value] | 0.000 | Vertical constraint active |
| Center Point | 0.75 | 0.25 | [Value] | [Value] | Representative interior point |